- Simple Distillation of a Volatile Liquid

Containing Non-volatile Impurities

Assemble the simple distillation apparatus as shown in the pictures below.  Use a 100 mL round-bottomed

flask for the still pot.

- Have your instructor check your apparatus for proper assembly.

Remove the Thermometer/adapter assembly from your apparatus, and insert your long-stem

plastic funnel.

Pour 50 mL of cyclohexane ( B.R. 80 - 82oC ) through the funnel into your apparatus.

- Note the color and clarity of your pure liquid on your laboratory data sheet.

Remove the funnel from your apparatus and add your magnetic stirring bar.

Replace the thermometer/ adapter assembly onto your apparatus.

- Have your instructor check your apparatus before you start heating.

Place a 50 mL graduated cylinder so that the tip of the condenser is centered over the neck

of the cylinder, as shown on the demonstration set-up.

- Turn the water on so that a gentle flow comes from the discharge end of the apparatus.

Statr stirring and begin heating the distillation flask on a setting of “4”.

As soon as the liquid begins to boil and the condensing vapors have reached the thermometer bulb,

regulate the heat supply so that distillation continues steadily at 1 drop per second.

- As soon as drops begin falling into the receiver, record the temperature from the thermometer in the

distillation head.  Record the temperature every four mL thereafter.

Discontinue heating when only 2-3 mL of your pure liquid remains in the distillation flask

- Pour the contents of your receiver into the recovery bottle.

- Draw a Temperature vs. Volume graph of your lab data on your laboratory report form.

- Note the boiling range of your pure liquid on your lab report form, and note the color and

clarity of your distillate.

Che 301         Su’2008

- Lab Data Sheet

- Title: Simple Distillation

- Name:___________________________________________	Locker #:___________

- Date:______________________________

- Lab Data

- Liquid Used:_________________________________

- Appearance of the liquid

( circle the appropriate description(s) )

Colorless   color               			  cloudy  clear

other comments______________________________________________________________

- Volume of Liquid Used:								          mL.

Temp. at which first drops enter the receiving vessel: 				     ___ oC

- Volume/Temp Data

| Vol (mL) | 4 | 8 | 12 | 16 | 20 | 24 | 28 | 32 | 36 | 40 | 44 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temp (oC) |  |  |  |  |  |  |  |  |  |  |  |